Flutter on Ubuntu Complete Installation Guide Ubuntu 22.04 LTS ¦ Flutter 3.41.4 ¦ Android SDK 36

# Overview

This guide walks you through a complete, error-free Flutter installation on Ubuntu 22.04 LTS — including the Android toolchain setup. Every command has been tested and corrected so you can execute them top to bottom without interruption.

| Requirement | Details |
| --- | --- |
| OS | Ubuntu 22.04 LTS (64-bit) |
| RAM | 8 GB minimum (16 GB recommended) |
| Disk | ~10 GB free space for Flutter + Android SDK |
| Internet | Required for downloads (~1.8 GB total) |
| Privileges | sudo access required |

# Step 1 — Install Flutter via Snap

Install Flutter using the Snap package manager with the --classic flag, which gives Flutter full system access needed for SDK management.

sudo snap install flutter --classic

Verify the installation and initialize Flutter (this downloads the SDK, ~1.4 GB):

flutter sdk-path

⚑ Note: This triggers Flutter's first-time initialization and downloads the stable release. The download takes several minutes depending on your connection speed.

# Step 2 — Set Persistent Environment Variables

This is a critical step that is often skipped. Environment variables set with export in a terminal session are lost when the terminal closes. To make them permanent, add them to ~/.bashrc.

echo 'export ANDROID_HOME=$HOME/Android/Sdk' >> ~/.bashrc

echo 'export PATH=$PATH:$ANDROID_HOME/platform-tools:$ANDROID_HOME/cmdline-tools/latest/bin' >> ~/.bashrc

source ~/.bashrc

⚑ Note: Always use source ~/.bashrc after editing it to apply changes to the current session without restarting the terminal.

# Step 3 — Install Android Command-Line Tools

Flutter requires the Android SDK command-line tools. Download and set them up in the correct directory structure that Flutter expects.

## 3.1 — Create SDK directory and download tools

mkdir -p ~/Android/Sdk/cmdline-tools

cd ~/Android/Sdk/cmdline-tools

wget https://dl.google.com/android/repository/commandlinetools-linux-11076708_latest.zip

unzip commandlinetools-linux-11076708_latest.zip

## 3.2 — Rename to the correct folder name

Flutter looks specifically for a folder named latest inside cmdline-tools. Rename the extracted folder:

mv cmdline-tools latest

⚑ Note: This renaming step is essential. Without it, Flutter cannot locate sdkmanager and will report 'Android sdkmanager not found'.

# Step 4 — Point Flutter to the Android SDK

Tell Flutter where your Android SDK lives:

flutter config --android-sdk $HOME/Android/Sdk

# Step 5 — Install Required Android SDK Packages

Install the correct platform and build-tools versions that Flutter 3.41.4 requires. Run all packages in a single command to avoid the 'command not found' error caused by line breaks.

sdkmanager "platform-tools" "platforms;android-36" "build-tools;28.0.3" "build-tools;34.0.0"

⚑ Note: Flutter 3.41.4 requires Android SDK 36 and BuildTools 28.0.3 as a minimum. Installing build-tools;34.0.0 alongside is recommended for compatibility with newer projects.

| Package | Required Version | Purpose |
| --- | --- | --- |
| platform-tools | Latest | adb, fastboot, etc. |
| platforms;android-36 | 36 | Android 16 SDK platform |
| build-tools;28.0.3 | 28.0.3 | Flutter minimum requirement |
| build-tools;34.0.0 | 34.0.0 | Recommended for new projects |

# Step 6 — Accept Android SDK Licenses

Android requires you to accept its SDK licenses before Flutter can use the toolchain:

flutter doctor --android-licenses

When prompted, type y and press Enter to accept each license. There are typically 6-7 licenses to accept.

# Step 7 — Verify the Installation

Run flutter doctor to confirm everything is set up correctly:

flutter doctor

Expected output — all green checkmarks:

[✓] Flutter (Channel stable, 3.41.4, ...)

[✓] Android toolchain - develop for Android devices (Android SDK version 36.x.x)

[✓] Chrome - develop for the web

[✓] Linux toolchain - develop for Linux desktop

[✓] Connected device

[✓] Network resources

# Quick Reference — All Commands

Copy and run all commands in order for a clean installation:

# 1. Install Flutter

sudo snap install flutter --classic

flutter sdk-path

# 2. Persist environment variables

echo 'export ANDROID_HOME=$HOME/Android/Sdk' >> ~/.bashrc

echo 'export PATH=$PATH:$ANDROID_HOME/platform-tools:$ANDROID_HOME/cmdline-tools/latest/bin' >> ~/.bashrc

source ~/.bashrc

# 3. Download Android command-line tools

mkdir -p ~/Android/Sdk/cmdline-tools

cd ~/Android/Sdk/cmdline-tools

wget https://dl.google.com/android/repository/commandlinetools-linux-11076708_latest.zip

unzip commandlinetools-linux-11076708_latest.zip

mv cmdline-tools latest

# 4. Configure Flutter SDK path

flutter config --android-sdk $HOME/Android/Sdk

# 5. Install SDK packages (all in one command)

sdkmanager "platform-tools" "platforms;android-36" "build-tools;28.0.3" "build-tools;34.0.0"

# 6. Accept licenses

flutter doctor --android-licenses

# 7. Verify

flutter doctor

# Troubleshooting

## Problem: 'Android sdkmanager not found'

Cause: The cmdline-tools folder was not renamed to latest, or the PATH was not exported persistently.

Fix:

ls ~/Android/Sdk/cmdline-tools/ # Should show a 'latest' folder

# If it shows 'cmdline-tools' instead, rename it:

mv ~/Android/Sdk/cmdline-tools/cmdline-tools ~/Android/Sdk/cmdline-tools/latest

source ~/.bashrc

## Problem: 'Unable to locate Android SDK'

Cause: Flutter's android-sdk config was not set, or ANDROID_HOME was not persisted.

Fix:

flutter config --android-sdk $HOME/Android/Sdk

source ~/.bashrc

flutter doctor

## Problem: 'Flutter requires Android SDK 36 and BuildTools 28.0.3'

Cause: Wrong SDK/build-tools versions were installed (e.g., android-34 instead of android-36).

Fix:

sdkmanager "platforms;android-36" "build-tools;28.0.3"

## Problem: 'build-tools;34.0.0: command not found'

Cause: The sdkmanager command was split across multiple lines in the terminal, causing only the first argument to be passed.

Fix: Always put all sdkmanager packages in a single unbroken command:

sdkmanager "platform-tools" "platforms;android-36" "build-tools;28.0.3" "build-tools;34.0.0"

## Problem: 'No valid Android SDK platforms found'

Cause: The platform was installed but build-tools were missing, making the toolchain incomplete.

Fix: Reinstall both together:

sdkmanager "platforms;android-36" "build-tools;28.0.3"

flutter doctor

Guide validated on Flutter 3.41.4 + Ubuntu 22.04 LTS For the latest Flutter version requirements, always check: flutter.dev/to/linux-android-setup